Standard Operating Procedure

Circular saw

|  | Dust Mask |  | Earmuffs/ Plugs |
| --- | --- | --- | --- |
|  | Safety Glasses |  | Close fitting Clothing |
|  | Safety Footwear |  | Long and loose hair must be contained or restrained |

### PRE-OPERATIONAL SAFETY CHECKS

- Never use electrical cord to lift, lower or carry the Circular saw.

- Check the Circular Saw body, guards, guides, and blades for damage before use.

- Check the lead and plug and that the tag is current.

- Ensure the blade is suitable for the task.

- Always disconnect from power source before changing the blade.

- Never hold piece being cut in your hand or across your leg. Secure the work piece to a stable platform.

- Adjust the cutting depth to the thickness of the workpiece. Less than a full tooth of the blade should be visible below the workpiece.

- Ensue you are familiar with the ON/OFF switch.

#### OPERATIONAL SAFETY CHECKS

- Always allow the Circular Saw to reach the maximum revolutions before starting cut.

- Do not start Circular Saw with the blade touching material.

- Keep hands and leads away from the Circular blade when cutting.

- Always hold the Circular Saw firmly and cut at a moderate rate to resist kickback forces.

- If the blade binds, release the trigger and hold the saw motionless in the material until the blade comes to a complete stop before removing the saw.

- Never place the Circular Saw down on a surface until the blade is completely stopped.

#### HOUSEKEEPING

- Switch off and leave the Circular Saw and floor in a safe, clean and tidy state.

- Make sure good housekeeping practices are in place to minimise any dust build-up.

## POTENTIAL HAZARDS

Exposure to Blade      Kick back         Ejected waste

Eye injury        Wood dust         Splinters        Cuts

## FORBIDDEN

Operating without the guard in place

Operating without the guard spring mechanism working correctly

Operating with any damage to the saw blade

Editing Instructions: note that the Zest Safety Logos can be deleted, and your own company logos inserted. Font Colours can be changed to your company colour palettes.

Disclaimer: This resource document has been created with the utmost care and accuracy, referencing the latest guidelines and information available at the time of production. However, businesses or individuals using this free resource are advised to independently verify the accuracy of the information provided. For comprehensive and tailored advice, it is recommended to engage a qualified health and safety advisor.

For further free H&S documents visit www.zestsafety.com