Standard Operating Procedure

Orbital Sander

|  | Dust Mask |  | Earmuffs/ Plugs |
| --- | --- | --- | --- |
|  | Safety Glasses |  | Close fitting Clothing |
|  | Safety Footwear |  | Long and loose hair must be contained or restrained |

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure this power tool has a suitable safe work area.

- Examine the power lead and plug for obvious damage and that the test date is current.

- Do not use dull or damaged abrasive sanding discs.

- Check the sanding disc is correctly fitted.

- Ensue you are familiar with the ON/OFF switch.

#### OPERATIONAL SAFETY CHECKS

#### 1. Do not plug in until all adjustments have been made.

#### 2. This sander is not suitable for wet sanding.

#### 3. Examine the material to be sanded for splits, loose knots and nails, etc.

#### 4.  Always select the correct abrasive and grade of disc.

#### 5. Sanding discs with holes must align with those on the rubber pad to allow for efficient dust collection.

#### 6. Always make sure a dust bag (or ducting) is fitted and emptied regularly.

#### 7. Ensure the work piece is well supported and secure.

#### 8. Ensure the power cord is well clear of the work piece.

#### 9. This sander is designed to be used with both hands. Keep fingers and hand away from moving parts.

#### 10. Allow the sander to reach operating speed, then apply load gradually, do not use excessive force.

#### 11. Avoid blocking ventilation slots.

#### 12. Avoid prolonged use and high pressures, this could overheat the motor.

#### 13. Be cautious of sanding too close to edges and corners.

#### HOUSEKEEPING

- Return this tool to the appropriate storage cupboard.

- Leave the work area in a safe, clean and tidy state.

- Make sure good housekeeping practices are in place to minimise any dust build-up.

## POTENTIAL HAZARDS

Moving, rotating and abrasive parts.     Electricity         Dust

Eye injury        Noise        Burns (friction)

Editing Instructions: note that the Zest Safety Logos can be deleted, and your own company logos inserted. Font Colours can be changed to your company colour palettes.

Disclaimer: This resource document has been created with the utmost care and accuracy, referencing the latest guidelines and information available at the time of production. However, businesses or individuals using this free resource are advised to independently verify the accuracy of the information provided. For comprehensive and tailored advice, it is recommended to engage a qualified health and safety advisor.

For further free H&S documents visit www.zestsafety.com