Standard Operating Procedure

Framing Nail Gun

|  | Long and loose hair must be contained or restrained |  | Earmuffs/ Plugs |
| --- | --- | --- | --- |
|  | Safety Glasses |  | Close fitting Clothing |
|  | Safety Footwear |

### PRE-OPERATIONAL SAFETY CHECKS

- Read tool manual cover to cover.

- Establish an appropriate exclusion zone when nailing.

- Ensure the actuation muzzle is serviceable and in place

- Check condition of the tool and do not operate if not functioning properly.

- Do not use the tool near any volatile or flammable materials.

#### OPERATIONAL SAFETY CHECKS

#### 1.  Always assume the tool contains fasteners.

#### 2.  Never point the tool at yourself or anyone with or without fasteners – no horseplay.

#### 3.  Do not actuate the tool unless the tool is placed firmly against the work piece.

#### 4.  Do not hold or carry the tool with a finger on the trigger.

#### 5.  Never operate a tool that has malfunctioned.

#### 7.  Never disable or remove the work contacting element.

#### 8.  Maintain good footing and balance.

#### 9.  Do not load fasteners with the trigger or work contacting element depressed.

#### 10.Never attempt to clear jams before removing the fuel cylinder.

#### 11.Keep your hands away from the area being fastened.

#### HOUSEKEEPING

- Do not store gas fuel cells above 48oC or 120oF this may cause a fire or explosion.

- Never puncture, incinerate, crush or refill fuel cells.

- Always remove fuel, battery cell and fasteners before storage.

- Keep the tool clean.

- Make sure good housekeeping practices are in place to minimise any dust build-up.

- Only charge battery on non-flammable surfaces.

- Do not cover battery with anything during charging

## POTENTIAL HAZARDS

Exposure to ejected nails      Compressed gases      Pinch and squash

Noise     Eye injuries     Fire or explosion during battery charging

Editing Instructions: note that the Zest Safety Logos can be deleted, and your own company logos inserted. Font Colours can be changed to your company colour palettes.

Disclaimer: This resource document has been created with the utmost care and accuracy, referencing the latest guidelines and information available at the time of production. However, businesses or individuals using this free resource are advised to independently verify the accuracy of the information provided. For comprehensive and tailored advice, it is recommended to engage a qualified health and safety advisor.

For further free H&S documents visit www.zestsafety.com