Standard Operating Procedure

Hammer Drill – cordless / corded

|  | Dust Mask When required |  | Earmuffs/ Plugs When required |
| --- | --- | --- | --- |
|  | Safety Glasses |  | Close fitting Clothing |
|  | Safety Footwear |  | Long and loose hair must be contained or restrained |

### PRE-OPERATIONAL SAFETY CHECKS

### 1. Never use electrical cord to lift, lower or carry the hammer Drill.

### 2. Check Hammer Drill body and cord and plug for damage and that the tag is current.

### 3. Select correct drill bit making sure it is clean and sharp.

### 4. Always use with side grip, holding firmly with both hands.

#### OPERATIONAL SAFETY CHECKS

#### 1. Fit and hand tighten drill bit into chuck then tighten securely with chuck key.

#### 2. Adjust speed change to “L for low or “H” for high and adjust variable speed to suit task.

#### 3. Ensure you are familiar with the location of the ON/OFF switch.

#### 4. Use firm pressure but do not force tool.

#### 5. Allow drill to stop completely before setting down or changing drill bit.

#### 6. When drilling into walls or floors or where “live” wires may be encountered, DO NOT TOUCH ANY METAL PARTS OF THE TOOL! Hold the tool by the insulated grasping surfaces only.

#### HOUSEKEEPING

#### 1. Switch off and leave Drill, bench and floor in a safe, clean and tidy state.

#### 2. Make sure good housekeeping practices are in place to minimise any dust build-up.

#### 3. Only charge battery on non-flammable surfaces

#### 4. Do not cover battery with anything during charging

## POTENTIAL HAZARDS

Exposure to moving & rotating components      Entanglement         Ejected waste

Eye injury       Wood/concrete chips      Wood/cement  dust            Braking drill bit

Electric shock

## FORBIDDEN

Using Drill for anything apart from its purpose.

Editing Instructions: note that the Zest Safety Logos can be deleted, and your own company logos inserted. Font Colours can be changed to your company colour palettes.

Disclaimer: This resource document has been created with the utmost care and accuracy, referencing the latest guidelines and information available at the time of production. However, businesses or individuals using this free resource are advised to independently verify the accuracy of the information provided. For comprehensive and tailored advice, it is recommended to engage a qualified health and safety advisor.

For further free H&S documents visit www.zestsafety.com