Standard Operating Procedure

Tile Cutter

|  | Dust Mask |  | Earmuffs/ Plugs |
| --- | --- | --- | --- |
|  | Safety Glasses |  | Close fitting Clothing |
|  | Safety Footwear |  | Long and loose hair must be contained or restrained |

### PRE-OPERATIONAL SAFETY CHECKS

- Check that the machine and surrounding area is clear of debris.

- Check the Blade is in good condition and suitable for the task.

- Check that safety guard is in place and operating correctly.

- Check that the sliding table is running freely.

- Check the lead and plug are in good condition and the Tag and Test is valid.

- Locate and make sure you are familiar with the ON/OFF trigger.

- Make sure the machine is on a stable platform and is secure.

- Ensure the water supply and hose connections are working.

#### OPERATIONAL SAFETY CHECKS

- Do not start the Machine with the Blade touching the stone/tile.

- Always allow the Blade to reach the maximum revolutions before starting cut.

- Ensure material is held firmly on table and in correct position.

- Turn on water and start machine.

- Ensure you keep your hands clear of the disc.

- Do Not force the Blade.

- Always stand to one side of Blade when cutting material in case of kickback.

- Ensure you keep your hands clear of the cutting line.

- After finishing the cut, wait until the disc stops rotating before returning to the start position.

- Never leave the machine running while unattended.

#### HOUSEKEEPING

- Switch off and leave the machine and floor in a safe, clean and tidy state, dispose of off cuts.

- Make sure you have turned off the water supply at source.

- Make sure good housekeeping practices are in place to minimise any dust build-up.

## POTENTIAL HAZARDS

Exposure to moving & rotating components         Pinch & squash         Ejected waste

Eye injury        Stone dust         Chips        Kick back        cuts

## FORBIDDEN

Cutting anything other than stone and ceramic.

Cutting irregular stock.

Cutting without the water on – creating dust

Editing Instructions: note that the Zest Safety Logos can be deleted, and your own company logos inserted. Font Colours can be changed to your company colour palettes.

Disclaimer: This resource document has been created with the utmost care and accuracy, referencing the latest guidelines and information available at the time of production. However, businesses or individuals using this free resource are advised to independently verify the accuracy of the information provided. For comprehensive and tailored advice, it is recommended to engage a qualified health and safety advisor.

For further free H&S documents visit www.zestsafety.com