Standard Operating Procedure

Tile Cutter

|  | Dust Mask when required. |  | Earmuffs/ Plugs |
| --- | --- | --- | --- |
|  | Safety Glasses |  | Close fitting Clothing |
|  | Face shield |  | Long and loose hair must be contained or restrained |
|  | Safety Footwear |

### PRE-OPERATIONAL SAFETY CHECKS

1.   Only to be used in designated areas.

2.   Be aware of others in your work area.

3.   Check the lead, machine and disc for damage and that the Test and Tag is current.

5.   Ensure the disc/blade guard is in place and depth adjustment set and secured.

6.   Ensure you are familiar with the ON/OFF switch.

#### OPERATIONAL SAFETY CHECKS

#### 1.   Plug into power only after all cutting disc/blade adjustments have been made.

#### 2.   Ensure your workpiece is firmly secured & supported.

#### 3.   Keep fingers, hands and clothing clear of the disc/blade.

#### 4.   Use Tile Cutter only in a designated work area – preferably behind a portable screen.

#### 5.   Be aware of flying hot sparks.  Hold the Tile Cutter so that any sparks fly away from you and anyone nearby, and away from all flammable materials.

#### 6.   Allow the Tile Cutter to reach operating speed, then apply load gradually.  Maintain a constant pace to avoid uneven surfaces.

#### 7.   Maintain complete control.  Always maintain a proper & steady footing.

#### 8.   Do not apply excessive force & avoid prolonged use.

#### 9.   Violent kick-back can occur, when cutting.

#### 10.  Wear rubber gloves if “live” wires could be encountered (for protection against shock).

#### 11.  Turn off after use.  Do not place the Tile Cutter down until the disc has stopped rotating.

#### HOUSEKEEPING

- Switch off and leave the machine bench and floor in a safe, clean and tidy state.

- Make sure good housekeeping practices are in place to minimise any dust build-up.

## POTENTIAL HAZARDS

Exposure to moving & rotating components         Burns         Noise

Eye injury        Hot Sparks         Chips and Dust      Electric shock

Editing Instructions: note that the Zest Safety Logos can be deleted, and your own company logos inserted. Font Colours can be changed to your company colour palettes.

Disclaimer: This resource document has been created with the utmost care and accuracy, referencing the latest guidelines and information available at the time of production. However, businesses or individuals using this free resource are advised to independently verify the accuracy of the information provided. For comprehensive and tailored advice, it is recommended to engage a qualified health and safety advisor.

For further free H&S documents visit www.zestsafety.com